Department of Electronic and Telecommunication Engineering

University of Moratuwa

EN3030 Circuits and Systems Design

Processor Design – Project Report

| T.M. Aqeel | 180039C |
| --- | --- |
| D.N.R. Dissanayake | 180153U |
| G.K.H. Nadeeshan | 180410G |
| R.T.N. Rathnayake | 180538F |

06th July 2022

Table of Contents

Abstract	3

1. Introduction	4

1.1 Processor Design	4

1.2 Central Processing Unit (CPU)	4

1.3 Microprocessor	4

1.4 Problem Statement	5

1.5 Proposed Solution	5

2. Designing the Processor	6

2.1 Datapath and Control Signals	6

2.2 Instruction Set	6

2.3 Microinstructions	6

2.4 Finite State Machines for Microinstructions	10

2.5 Modules and Components	10

2.5.1 Machine	10

2.5.2 Inside of the machine	11

2.5.3 Processor	11

2.5.4 Modules Inside the Processor	12

2.5.5 ALU	12

2.5.6 Control Unit	13

2.5.7 Accumulator	14

2.5.8 Register Which Can Be Incremented	15

2.5.9 Register Which Cannot Be Incremented	15

2.5.10 Data Bus	15

2.5.11 Data Memory	16

2.5.12 Instruction Memory	16

3. Algorithm and Design Considerations	17

3.1 Filtering & Down-sampling	17

3.2 Low Pass Filter	17

3.3 Down-sampling	18

3.4	Data Processing Techniques	19

4.	RTL Design Simulation	21

5.	References	22

6. Appendix	23

6.1 Creating input file (Python)	23

6.2 Assembly code	23

6.2.1 Compiler to binary values (Python)	26

6.2.2 Compiler to ram (Python)	28

6.3 Verilog codes for processor design	29

6.3.1 Accumulator	29

6.3.2 ALU	29

6.3.3 Bus	30

6.3.4 State machine	30

6.3.5 Data memory	41

6.3.6 Instruction memory	41

6.3.7 Top module	45

6.3.8 Processor	46

6.3.9 Registers without increment	48

6.3.10 Registers with increment	48

6.3.11 Test bench	48

6.4 Generating images	50

6.4.1 By processor implementation	50

6.4.2 By Python implementation	51

# Abstract

Custom processors are widely used in the electronic world. Those processors can do a specific task with better performance and good reliability. This report contains a detailed discussion of the processor designed for the given task with designing steps and the simulated outputs of the processor. The method and the theoretical approaches are included as well as Verilog code, Assembly code, and ISA are attached as reference.

# 1. Introduction

## 1.1 Processor Design

The purpose of this project is to filter and down sample a 256*256 px image to 128*128 px image. A task specific processor can do this task with a good reliability. The designed processor can be simulated using Verilog Hardware Description Language (HDL) and the implementations are done using Vivado 2018.2. This report contains the Microprocessor and CPU design, the test code that are used to test the filtering and down sampling of a given image.

## 1.2 Central Processing Unit (CPU)

The central processing unit includes a control unit, arithmetic and logical unit, and memory unit to perform a program. These electronic elements need to work correctly to carry out the instructions of a computer program. The Processing Unit and the Control Unit (CU) are mainly identified as the Processor of a CPU by distinguishing these core elements of a computer from external components such as main memory and I/O circuitry.

## 1.3 Microprocessor

A microprocessor is a single integrated circuit (IC) which is a multipurpose, clock-driven, register-based, digital-integrated circuit that takes binary inputs, and processes them according to the instruction stored in its memory. And it provides results as output. Microprocessors can be designed to perform a special task not to do multiple tasks. These customizations will lead to achieve more reliable results with good efficiency.

## 1.4 Problem Statement

The main task of the given project is to design a processor to down sample a 256*256px image. According to the given task, the main requirements for the designing processor are filtering and down sampling a given image.

The followings are the constraints that were selected for the processor design.

- Original Image - 256*256 px image

- Down Sampling factor - 2

- Image type - gray

## 1.5 Proposed Solution

Give the output data to convert the data stream into an image

Down sample the input image

Filter the input image

Give the input data stream to the memory unit

Convert the pixel data of the original image

A python program with open cv library is used to convert the pixel data into a binary data stream.

In this step, it needs to place the input binary data stream file into sim folder. Because the memory unit takes the input from this folder

After receiving this data into the processor, it stores data in its memory unit, so that data can be accessed when required. When data reception is completed, the processor starts to filter the pixel values according to a filtering algorithm. Filtered data are again stored in the memory unit. After filtering all the pixel values, the processor starts down sampling pixel by pixel. Output data are again stored in the memory unit.

A python program with open cv library is used to convert the binary data into an image

# 2. Designing the Processor

## 2.1 Datapath and Control Signals

- Click here

## 2.2 Instruction Set

There are 31 instructions that can be used by the assembly programmer. Some instructions have a 16-bit operand, and most instructions are zero operand instructions. 8 bits are used for opcode. Following table explains the operations of each instruction.

| Instruction | Opcode | Operand (16 bit) | Operation |
| --- | --- | --- | --- |
| loadac | 0000 0100 | - | ac <= DM[ac] |
| movacr | 0000 1000 | - | r <= ac |
| movacr1 | 0000 1001 | - | r1 <= ac |
| movacr2 | 0000 1010 | - | r2 <= ac |
| movacr3 | 0000 1011 | - | r3 <= ac |
| movacr4 | 0000 1100 | - | r4 <= ac |
| movacr5 | 0000 1101 | - | r5 <= ac |
| movacdar | 0000 1110 | - | dar <= ac |
| movrac | 0000 1111 | - | ac <= r |
| movr1ac | 0001 0000 | - | ac <= r1 |
| movr2ac | 0001 0001 | - | ac <= r2 |
| movr3ac | 0001 0010 | - | ac <= r3 |
| movr4ac | 0001 0011 | - | ac <= r4 |
| movr5ac | 0001 0100 | - | ac <= r5 |
| movdarac | 0001 0101 | - | ac <= dar |
| stac | 0001 0110 | - | DM[ac] <= ac |
| add | 0001 1001 | - | ac <= ac + r |
| sub | 0001 1011 | - | ac <= ac – r |
| lshift | 0001 1101 | - | ac <= r << ac |
| rshift | 0001 1111 | - | ac <= r >> ac |
| incac | 0010 0001 | - | ac <= ac + 1 |
| incdar | 0010 0010 | - | dar <= dar +1 |
| incr1 | 0010 0011 | - | r1 <= r1 +1 |
| incr2 | 0010 0100 | - | r2 <= r2 +1 |
| incr3 | 0010 0101 | - | r3 <= r3 + 1 |
| loadim | 0010 0110 | imm | ac <= imm |
| jumpz | 0010 1001 | imm | if z == 1, go to instruction IM[imm] |
| jumpnz | 0011 0000 | imm | if z == 0, go to instruction IM[imm] |
| jump | 0011 0001 | imm | go to instruction IM[imm] |
| nop | 0011 0010 | - | no operation |
| endop | 0011 0011 | - | end process |

## 2.3 Microinstructions

These microinstructions are used for finite state machine in the control unit. one microinstruction can be operated in one clock cycle. Following table explains operations of each microinstruction.

| Instruction (8bits) | Opcode (decimal) | Microinstruction (6bits) | Operation |
| --- | --- | --- | --- |
| idle (internal) | - | idle | goto fetch1 if status == 1 else idle |
| fetch (internal) | - | fetch1 | read IMEM |
|  |  |  | write ir |
|  |  |  | goto fetch2 |
|  |  | fetch2 | read IMEM |
|  |  |  | write ir |
|  |  |  | goto fetch3 |
|  |  | fetch3 | read IMEM |
|  |  |  | goto ir[5:0] |
| loadac | 4 | loadac1 | read ac |
|  |  |  | write dar |
|  |  |  | goto loadac2 |
|  |  | loadac2 | read ac |
|  |  |  | wrirte dar |
|  |  |  | goto loadac3 |
|  |  | loadac3 | read DMEM |
|  |  |  | write ac |
|  |  |  | goto loadac4 |
|  |  | loadac4 | read DMEM |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movacr | 8 | movacr | read ac |
|  |  |  | write r |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movacr1 | 9 | moveacr1 | read ac |
|  |  |  | write r1 |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movacr2 | 10 | moveacr2 | read ac |
|  |  |  | write r2 |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movacr3 | 11 | moveacr3 | read ac |
|  |  |  | write r3 |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movacr4 | 12 | moveacr4 | read ac |
|  |  |  | write r4 |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movacr5 | 13 | moveacr5 | read ac |
|  |  |  | write r5 |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movacdar | 14 | moveacdar | read ac |
|  |  |  | write dar |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movrac | 15 | moverac | read r |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movr1ac | 16 | mover1ac | read r1 |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movr2ac | 17 | mover2ac | read r2 |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movr3ac | 18 | mover3ac | read r3 |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movr4ac | 19 | mover4ac | read r4 |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movr5ac | 20 | mover5ac | read r5 |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| movdarac | 21 | movedarac | read dar |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| stac | 22 | stac1 | read ac |
|  |  |  | goto stac2 |
|  |  | stac2 | read ac |
|  |  |  | write dm |
|  |  |  | goto stac3 |
|  |  | stac3 | read ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| add | 25 | add1 | add alu |
|  |  |  | goto add2 |
|  |  | add2 | add alu |
|  |  |  | write alu_to_ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| sub | 27 | sub1 | sub alu |
|  |  |  | goto sub2 |
|  |  | sub2 | sub alu |
|  |  |  | write alu_to_ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| lshift | 29 | lshift1 | lshift alu |
|  |  |  | goto lshift2 |
|  |  | lshift2 | lshift alu |
|  |  |  | write alu_to_ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| rshift | 31 | rshift1 | rshift alu |
|  |  |  | goto rshift2 |
|  |  | rshift2 | rshift alu |
|  |  |  | write alu_to_ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| incac | 33 | incac | increment ac |
|  |  |  | incremtnt pc |
|  |  |  | goto fetch1 |
| incdar | 34 | incdar | increment dar |
|  |  |  | incremtnt pc |
|  |  |  | goto fetch1 |
| incr1 | 35 | incr1 | increment r1 |
|  |  |  | incremtnt pc |
|  |  |  | goto fetch1 |
| incr2 | 36 | incr2 | increment r2 |
|  |  |  | incremtnt pc |
|  |  |  | goto fetch1 |
| incr3 | 37 | incr3 | increment r3 |
|  |  |  | incremtnt pc |
|  |  |  | goto fetch1 |
| loadim | 38 | loadim1 | increment pc |
|  |  |  | goto loadim2 |
|  |  | loadim2 | read IMEM |
|  |  |  | goto loadim3 |
|  |  | loadim3 | read IMEM |
|  |  |  | write ac |
|  |  |  | increment pc |
|  |  |  | goto fetch1 |
| jump | 49 | jump | increment pc |
|  |  |  | goto jumpz2 |
| jumpz | 41 | jumpz1 | increment pc |
|  |  |  | goto jumpz2 if z == 1 else jumpz6 |
|  |  | jumpz2 | read IMEM |
|  |  |  | write pc |
|  |  |  | goto jumpz3 |
|  |  | jumpz3 | read IMEM |
|  |  |  | write pc |
|  |  |  | goto jumpz4 |
|  |  | jumpz4 | read IMEM |
|  |  |  | write ir |
|  |  |  | goto jumpz5 |
|  |  | jumpz5 | read IMEM |
|  |  |  | write ir |
|  |  |  | goto fetch3 |
|  |  | jumpz6 | goto jumpz7 |
|  |  | jumpz7 | increment pc |
|  |  |  | goto jumpz4 |
| jumpnz | 48 | jumpnz1 | increment pc |
|  |  |  | goto jumpz3 if z == 0 else goto jumpz7 |
| endop | 51 | endop | read DMEM |
|  |  |  | goto endop |
| nop | 50 | nop | increment pc |
|  |  |  | goto fetch1 |

## 2.4 Finite State Machines for Microinstructions

- Click here

## 2.5 Modules and Components

### 2.5.1 Machine

There are three phases in this machine.

- Load the image – Phase 1

- Processing – Phase 2

- Get the processed image – Phase 3

In phase 1, we have to give the image data to data_in and memory address to data_addr_in. Then processing will be done. After phase 2 is done, end_process will be asserted. In phase 3, we can get the output image data from the data_out.

| Inputs | Outputs |
| --- | --- |
| clk – clock | end_process – signal that indicate process has ended |
| status – control signal for change phases (input image data, process data, output data) | data_out – ouput image data |
| data_in – input image data |  |
| data_addr_in – input data memory address |  |

| status | 10 | 01 | 11 |
| --- | --- | --- | --- |
| phase | 1 – load image | 2 – process | 3 – output image |

### 2.5.2 Inside of the machine

### 2.5.3 Processor

| Inputs | Outputs |
| --- | --- |
| clk – clock | end_process – signal that indicate process has ended |
| status – control signal for change phases (input image data, process data, output data) | dm_en – data memory write enable signal |
| dm_out – output of data memory | im_en – instruction memory write enable signal |
| im_out – output of instruction memory | pc_out – instruction memory address |
| data_in – input image data | dar_out – data memory address |
| data_addr_in – input data memory address | bus_out – data output for data memory |

- Inside of the Processor – Click here

### 2.5.4 Modules Inside the Processor

Arithmetic & Logic Unit (ALU) – Do arithmetic and logic operations in the processor

Control Unit – Do all the controlling in the processor. Implemented as a finite state machine.

Special purpose registers

  - Accumulator (ac) – input for the ALU, store ALU output

  - Program counter (pc) – directed to the next instruction to be fetched

  - Instruction register (ir) – store the instruction fetched from the instruction memory

  - Data address register (dar) – keep the address of the store location of the data memory. Loading from data memory and storing to data memory are being done using this address

  - r register – one of the inputs for the ALU

General purpose registers

  - r1, r2, r3 – general purpose registers which can be incremented

  - r4, r5 – general purpose registers which cannot be incremented

### 2.5.5 ALU

ALU is responsible for all the arithmetic and logic operations. Inputs are taken from ac and r registers and output is stored in ac register. We are using 3-bit alu_op even though there are only four operations since it is better to have some improvement capabilities. z will be 1 if the alu_out is zero. z will be 0 if the alu_out is nonzero.

| alu_op | Operation | Description |
| --- | --- | --- |
| 001 | Addition | Alu_out <= in1 + in2 |
| 010 | Subtraction | Alu_out <= in2 – in1 |
| 011 | Left shift | Alu_out <= in1 << in2 |
| 100 | Right shift | Alu_out <= in1 >> in2 |

### 2.5.6 Control Unit

Control Unit is responsible for all the control signals in the processor. Here we are implemented this using a finite state machine.

| Inputs | Outputs |
| --- | --- |
| clk – clock | end_process – signal that indicate process has ended |
| status – control signal for change phases (input image data, process data, output data) | read_en – 4 bit read enable signal |
| instruction – instruction from the instruction register | write_en – 16 bit write enable signal |
| z – z output from the alu | inc_en – 8 bit increment enable signal |
|  | alu_op – ALU opcode |

#### Read Enable – read_en

read_en is the control signal that is responsible for all the readings in the processor. Read values are written to the data bus. We cannot read from multiple registers and write them to the same bus. Hence, we are using 4 bit signal such that only one register can be read at one time.

| Decimal Value | Register |
| --- | --- |
| 0 | - |
| 1 | dar |
| 2 | ac |
| 3 | r |
| 4 | r1 |
| 5 | r2 |
| 6 | r3 |
| 7 | r4 |
| 8 | r5 |
| 9 | DM |
| 10 | IM |

#### Write Enable – write_en

write_en is the control signal that is responsible for all the writings in the processor. Unlike the reading, we can write data to multiple locations at the same time. Therefore, we are using 16-bit signal and each bit has assigned to separate location. We can write those location by setting those bits to 1.

| Bit 15 N/A | Bit 14 N/A | Bit 13 alu to ac | Bit 12 IM | Bit 11 DM | Bit 10 r1 | Bit 9 r2 | Bit 8 r3 | Bit 7 r4 | Bit 6 r5 | Bit 5 r | Bit 4 ac | Bit 3 ir | Bit 2 dar | Bit 1 pc | Bit 0 N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### Increment Enable – inc_en

inc_en is the control signal that is responsible for all the increments in the processor. Same as the writing, we can increment multiple registers at the same time.

| Bit 7 N/A | Bit 6 r3 | Bit 5 r2 | Bit 4 r1 | Bit 3 dar | Bit 2 ac | Bit 1 pc | Bit 0 N/A |
| --- | --- | --- | --- | --- | --- | --- | --- |

### 2.5.7 Accumulator

Accumulator is a special purpose register which has two data inputs. One is from the data bus and the other one is from the ALU output. There are two write enable signals to indicate from which input the writing should be done. This is one of the inputs to the ALU.

### 2.5.8 Register Which Can Be Incremented

pc, dar, r1, r2, r3 are registers of this type. The values in those registers can be incremented without using the ALU.

### 2.5.9 Register Which Cannot Be Incremented

ir, r, r4, r5 are registers of this type. The values in those registers cannot be incremented without using the ALU.

### 2.5.10 Data Bus

This is a 16-bit path which connects all the necessary components that needed to the data flow.

### 2.5.11 Data Memory

This is the main memory which data is stored. The image should be stored here before the processing begins. This memory contains 65536 locations of 8 bits long. Since we are using 256*256 image, 65536 pixel values are needed to be stored and that is why 65536 locations. Since the pixel value range is 0-255, we need 8-bit long memory to store that.

### 2.5.12 Instruction Memory

This is for storing the instructions. This is a read only memory. The processor cannot write into instruction memory during the processing. This can be written only when we need to load a new programme.

# 3. Algorithm and Design Considerations

## 3.1 Filtering & Down-sampling

Processor is designed to down sample an image. Down sampling is the reduction in spatial resolution while keeping the same two-dimensional (2D) representation. To down sample following steps are to be followed.

## 3.2 Low Pass Filter

Prior to down sampling an image, it should be low pass filtered to reduce the effects of aliasing caused by the high frequency components in the image. For the filtering purpose, there are mainly two types of low pass filtering kernels.

Although it is easy to perform addition using normalizing kernel, as it should be divisible by 9, we are not going to use this kernel for low pass filtering.

In Gaussian Kernel, shifting is needed to be done before addition. 16 is a number which is a power of 2, (16=24).  So, it can be easily shifted when we use Gaussian Kernel. The filtering algorithm is based on the 3x3 Gaussian kernel where the middle pixel location is considered as (0,0) and the standard deviation is 1 in the below equation and then approximating it to discrete numbers.

Gx,y=12πσ2e-x2+y22σ2

The Gaussian filtering can be used to blur the images and to remove the noise present. Here a 3×3 matrix is used mainly because 99% of the distribution fall within 3 standard deviations. The central pixel in Gaussian kernel has a higher weighting than the others on the sides.

|  |  |  |  |  | .. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | .. |  |  |
|  |  |  |  |  | .. |  |  |
|  |  |  |  |  | .. |  |  |
|  |  |  |  |  | .. |  |  |
| .. | .. | .. | .. | .. |  |  |  |

Now this center pixel is overlapped with the image pixel and a weighted summation value is calculated and it is divided by 16, which is the total weight of the kernel.

The kernel will calculate the average value first and stores it in the red colored pixel box. Then, it will move to the right to calculate the next average value. As per the figure it will move to the blue colored pixel box. After calculating all the average values in the right and storing it in that pixel, the kernel will move to the next row, which is marked in the yellow color. Same as earlier it will move right and will come to next row to filter all the pixel values. Finally, it will end up in the green colored pixel box. The gray colored pixel boxes remain unfiltered as it will not a big concern for down sampling.

## 3.3 Down-sampling

Down sampling reduces the number of pixels in image by a given down sampling factor. Here, the objective is to down sample an image by a down sampling factor 2. We are going to consider an image with the size 256×256. So, the resultant image size is 128×128. For that we are going to consider one value per four pixels from the above filtered image.

## Data Processing Techniques

The very first step of our task is to store the data included in the pixels of the 256x256 image into a text file. This can be done using the python code ‘input_create.py’. All the pixel values of the image are transformed to an array and the binary values of them are written in a text file. Data arrangement in the image and the text file is given below.

Then for the simulation of filtering and down sampling, the binary values in the text file are used as the input. From the simulation we can obtain the filtered output text file or the down sampled output text file, from considering the operation.

By using this filtered and down sampled text files as data files and by executing the ‘show.py’ python code, the binary values in the text files are stored in an array. After that this array is transformed into a matrix and then the filtered and down sampled images are obtained.

For the evaluation of the performance of our processor, a python code is used for filtering and down sampling. Here a comparison is made between the image data arrays of down sampled images from the designed processor and the python code execution by taking their difference. The error calculated from the above procedure is zero, which concludes that the designed processor is as per our expectations. Below images show the results from above mentioned methods.

Original, filtered and down sampled images from the processor

Original, filtered and down sampled images from the python code implementation

# RTL Design Simulation

Processor with memory modules

Processor without memory modules

# References

- https://youtube.com/playlist?list=PL5Q2soXY2Zi97Ya5DEUpMpO2bbAoaG7c6

- https://www.chipverify.com/verilog/verilog-tutorial

- https://www.geeksforgeeks.org/apply-a-gauss-filter-to-an-image-with-python/

- https://www.simplilearn.com/image-processing-article

All the source codes and other files can be found in this GitHub repository. https://github.com/TharukaN17/Processor_Design

# 6. Appendix

## 6.1 Creating input file (Python)

import cv2

image = cv2.imread('test.png', cv2.IMREAD_GRAYSCALE)

cv2.imshow('original', image)

cv2.waitKey()

image2 = image.ravel()

print(image2)

with open('input.txt', 'w') as f:

f.write('\n'.join("{0:b}".format(int(e)) for e in image2))

## 6.2 Assembly code

- loadim

- 16'd257       // Starting from 257th bit (2nd row, 2nd bit)

- movacr1

- movr1ac      // Start of the filtering loop

- loadac

- movacr        // R <- middle

- loadim

- 8'd2              // To multiply by 4

- lshift

- movacr4      // R4 <- middle*4

- loadim

- 8'd1              // To get right and left bits

- movacr

- movr1ac

- add

- loadac

- movacr5      // R5 <- right

- movr1ac

- sub

- loadac

- movacr

- movr5ac

- add

- movacr5      // R5 <- right + left

- loadim

- 16'd256       // To get bottom bit

- movacr

- movr1ac

- add

- loadac

- movacr

- movr5ac

- add

- movacr5     // R5 <- right + left + bottom

- loadim

- 16'd256      // To get top bit

- movacr

- movr1ac

- sub

- loadac

- movacr

- movr5ac

- add

- movacr       // R <- right + left + bottom + top

- loadim

- 8'd1             // To multiply by 2

- lshift

- movacr       // R <- (right + left + bottom + top)*2

- movr4ac

- add

- movacr4     // R4 <- middle*4 + (right + left + bottom + top)*2

- loadim

- 16'd257      // To get bottom_right and top_left bits

- movacr

- movr1ac

- add

- loadac

- movacr5     // R5 <- bottom_right

- movr1ac

- sub

- loadac

- movacr

- movr5ac

- add

- movacr5     // R5 <- bottom_right + top_left

- loadim

- 8'd255        // To get bottom_left bit

- movacr

- movr1ac

- add

- loadac

- movacr

- movr5ac

- add

- movacr5     // R5 <- bottom_right + top_left + bottom_left

- loadim

- 8'd255        // To get top_right bit

- movacr

- movr1ac

- sub

- loadac

- movacr

- movr5ac

- add

- movacr       // R <- bottom_right + top_left + bottom_left + top_right

- movr4ac

- add

- movacr       // R <- middle*4 + (right+left+bottom+top)*2 + (bottom_right+top_left+bottom_left+top_right)

- loadim

- 8'd4            // To divide by 16

- rshift

- movacr4    // R4 <- (middle*4 + (right+left+bottom+top)*2 + (bottom_right+top_left+bottom_left+top_right))/16

- loadim

- 16'd257     // To find store location

- movacr

- movr1ac

- sub

- movacdar

- movr4ac

- stac

- loadim

- 16'd65278   // To check end of the image ((256*254) + (255-1))

- movacr

- movr1ac

- sub

- jumpz

- 8'd127          // Go to the start of downscaling

- loadim

- 8'd253          // To check end of the row

- movacr

- movr2ac

- sub

- jumpz

- 8'd118

- incr1

- incr2

- jump

- 8'd3

- incr1

- incr1

- incr1

- loadim

- 8'd0

- movacr2

- jump

- 8'd3            // Go to the start of the filtering loop

- endop

- loadim        // Start of downscaling

- 8'd0

- movacr1     // R1 <- 0,   R1 = Keep track of current pixel

- movacr2     // R2 <- 0,   R2 = Keep track of current row pixel

- movacr3     // R3 <- 0,   R3 = Keep track of store location

- movr1ac     // Start of the downscaling loop

- loadac

- movacr4

- movr3ac

- movacdar

- movr4ac

- stac

- movr1ac

- movacr

- loadim

- 16'd65278   // To check end of the image ((256*254) + (255-1))

- sub

- jumpz

- 8'd172          // Go to end process

- movr2ac

- movacr

- loadim

- 8'd254          // To check end of the row

- sub

- jumpz

- 8'd160

- incr2

- incr2

- incr1

- incr1

- incr3

- jump

- 8'd132        // Go to the start of the downscaling loop

- loadim

- 8'd0

- movacr2

- loadim

- 16'd258      // To skip a row (256 + 2)

- movacr

- movr1ac

- add

- movacr1

- incr3

- jump

- 8'd132         // Go to the start of the downscaling loop

- endop         // End process

### 6.2.1 Compiler to binary values (Python)

compDict = {

"loadac"    : 4,

"movacr"    : 8,

"movacr1"   : 9,

"movacr2"   : 10,

"movacr3"   : 11,

"movacr4"   : 12,

"movacr5"   : 13,

"movacdar"  : 14,

"movrac"    : 15,

"movr1ac"   : 16,

"movr2ac"   : 17,

"movr3ac"   : 18,

"movr4ac"   : 19,

"movr5ac"   : 20,

"movdarac"  : 21,

"stac"      : 22,

"add"       : 25,

"sub"       : 27,

"lshift"    : 29,

"rshift"    : 31,

"incac"     : 33,

"incdar"    : 34,

"incr1"     : 35,

"incr2"     : 36,

"incr3"     : 37,

"loadim"    : 38,

"jumpz"     : 41,

"jumpnz"    : 48,

"jump"      : 49,

"nop"       : 50,

"endop"     : 51

}

instructionArray = open("Assembly.txt").read().splitlines()

outputFile = open("output_bin.txt", "w")

lines = []

comment = "//"

for i, instruction in enumerate(instructionArray):

if comment in instruction:

if comment in instruction[:3]:

continue

else:

instruction = instruction.split(comment)[0]

instruction = instruction.strip();

if instruction in compDict:

binaryValue = bin(compDict[instruction])[2:]

length = 8

else:

val = instruction.split("'d")

binaryValue = bin(int(val[1]))[2:]

length = int(val[0])

writeValue = (length-len(binaryValue))*'0'+binaryValue

lines.append(writeValue)

outputFile.writelines("\n".join(lines))

outputFile.close()

### 6.2.2 Compiler to ram (Python)

compDict = {

"loadac"    : 4,

"movacr"    : 8,

"movacr1"   : 9,

"movacr2"   : 10,

"movacr3"   : 11,

"movacr4"   : 12,

"movacr5"   : 13,

"movacdar"  : 14,

"movrac"    : 15,

"movr1ac"   : 16,

"movr2ac"   : 17,

"movr3ac"   : 18,

"movr4ac"   : 19,

"movr5ac"   : 20,

"movdarac"  : 21,

"stac"      : 22,

"add"       : 25,

"sub"       : 27,

"lshift"    : 29,

"rshift"    : 31,

"incac"     : 33,

"incdar"    : 34,

"incr1"     : 35,

"incr2"     : 36,

"incr3"     : 37,

"loadim"    : 38,

"jumpz"     : 41,

"jumpnz"    : 48,

"jump"      : 49,

"nop"       : 50,

"endop"     : 51

}

instructionArray = open("Assembly.txt").read().splitlines()

outputFile = open("output_ram.txt", "w")

lines = []

comment = "//"

for i, instruction in enumerate(instructionArray):

if comment in instruction:

if comment in instruction[:3]:

continue

else:

instruction = instruction.split(comment)[0]

instruction = instruction.strip()

if instruction in compDict:

value = "8'd"+str(compDict[instruction])

else:

value = instruction

writeValue = 'ram['+str(len(lines))+'] = '+value+';'

lines.append(writeValue)

outputFile.writelines("\n".join(lines))

outputFile.close()

## 6.3 Verilog codes for processor design

### 6.3.1 Accumulator

module ac(

input               clk, write_en, alu_to_ac,inc_en,

input       [15:0]  data_in, alu_out,

output reg  [15:0]  data_out = 16'd0

);

always @(posedge clk) begin

if (inc_en == 1)

data_out <= data_out + 16'd1;

else if (write_en == 1)

data_out <= data_in;

else if (alu_to_ac == 1)

data_out <= alu_out;

end

endmodule

### 6.3.2 ALU

module alu (

input               clk,

input       [15:0]  in1, in2,

input       [2:0]   alu_op,

output reg  [15:0]  alu_out,

output reg          z

);

always @(posedge clk)

begin

case(alu_op)

3'd1: alu_out <= in1 + in2;

3'd2: alu_out <= in2 - in1;

3'd3: alu_out <= in1 << in2;

3'd4: alu_out <= in1 >> in2;

endcase

if (alu_out == 0)

z <= 1;

else

z <= 0;

end

endmodule

### 6.3.3 Bus

module bus(

input               clk,

input       [3:0]   read_en,

input       [15:0]  r1,r2,r3,r4,r5,r,dar,ac,im,

input       [7:0]   dm,

output reg  [15:0]  bus_out

);

always @(*) begin

case (read_en)

4'd1:    bus_out <= dar;

4'd2:    bus_out <= ac;

4'd3:    bus_out <= r;

4'd4:    bus_out <= r1;

4'd5:    bus_out <= r2;

4'd6:    bus_out <= r3;

4'd7:    bus_out <= r4;

4'd8:    bus_out <= r5;

4'd9:    bus_out <= dm + 16'd0;

4'd10:   bus_out <= im;

default: bus_out <= 16'd0;

endcase

end

endmodule

### 6.3.4 State machine

module control(

input             clk, z,

input      [15:0] instruction,

input      [1:0]  status,

output reg [2:0]  alu_op,

output reg [7:0]  inc_en,

output reg [15:0] write_en,

output reg [3:0]  read_en,

output reg        end_process

);

reg [5:0] present = 6'd0;

reg [5:0] next    = 6'd0;

parameter

idle       = 6'd0,

fetch1     = 6'd1,

fetch2     = 6'd2,

fetch3     = 6'd3,

loadac1    = 6'd4,

loadac2    = 6'd5,

loadac3    = 6'd6,

loadac4    = 6'd7,

movacr     = 6'd8,

movacr1    = 6'd9,

movacr2    = 6'd10,

movacr3    = 6'd11,

movacr4    = 6'd12,

movacr5    = 6'd13,

movacdar   = 6'd14,

movrac     = 6'd15,

movr1ac    = 6'd16,

movr2ac    = 6'd17,

movr3ac    = 6'd18,

movr4ac    = 6'd19,

movr5ac    = 6'd20,

movdarac   = 6'd21,

stac1      = 6'd22,

stac2      = 6'd23,

stac3      = 6'd24,

add1       = 6'd25,

add2       = 6'd26,

sub1       = 6'd27,

sub2       = 6'd28,

lshift1    = 6'd29,

lshift2    = 6'd30,

rshift1    = 6'd31,

rshift2    = 6'd32,

incac      = 6'd33,

incdar     = 6'd34,

incr1      = 6'd35,

incr2      = 6'd36,

incr3      = 6'd37,

loadim1    = 6'd38,

loadim2    = 6'd39,

loadim3    = 6'd40,

jumpz1     = 6'd41,

jumpz2     = 6'd42,

jumpz3     = 6'd43,

jumpz4     = 6'd44,

jumpz5     = 6'd45,

jumpz6     = 6'd46,

jumpz7     = 6'd47,

jumpnz1    = 6'd48,

jump       = 6'd49,

nop        = 6'd50,

endop      = 6'd51;

always @(posedge clk)

present <= next;

always @(posedge clk) begin

if (present == endop)

end_process <= 1'd1;

else

end_process <= 1'd0;

end

always @(present or z or instruction or status)

case(present)

idle: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

if (status == 2'b01)

next <= fetch1;

else

next <= idle;

end

fetch1: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000001000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= fetch2;

end

fetch2: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000001000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= fetch3;

end

fetch3: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= instruction [5:0];

end

loadac1: begin

read_en  <=  4'd2;

write_en <= 16'b0000000000000100;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= loadac2;

end

loadac2 : begin

read_en  <=  4'd2;

write_en <= 16'b0000000000000100;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= loadac3;

end

loadac3 : begin

read_en  <=  4'd9;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= loadac4;

end

loadac4: begin

read_en  <=  4'd9;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movacr: begin

read_en  <=  4'd2;

write_en <= 16'b0000000000100000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movacr1: begin

read_en  <=  4'd2;

write_en <= 16'b0000000001000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movacr2: begin

read_en  <=  4'd2;

write_en <= 16'b0000000010000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movacr3: begin

read_en  <=  4'd2;

write_en <= 16'b0000000100000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movacr4: begin

read_en  <=  4'd2;

write_en <= 16'b0000001000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movacr5: begin

read_en  <=  4'd2;

write_en <= 16'b0000010000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movacdar : begin

read_en  <=  4'd2;

write_en <= 16'b0000000000000100;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movrac: begin

read_en  <=  4'd3;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movr1ac: begin

read_en  <=  4'd4;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movr2ac: begin

read_en  <=  4'd5;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movr3ac: begin

read_en  <=  4'd6;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movr4ac: begin

read_en  <=  4'd7;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movr5ac: begin

read_en  <=  4'd8;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

movdarac : begin

read_en  <=  4'd1;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

stac1: begin

read_en  <=  4'd2;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= stac2;

end

stac2: begin

read_en  <=  4'd2;

write_en <= 16'b0000100000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= stac3;

end

stac3: begin

read_en  <=  4'd2;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

add1: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd1;

next     <= add2;

end

add2: begin

read_en  <=  4'd0;

write_en <= 16'b0010000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd1;

next     <= fetch1;

end

sub1: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd2;

next     <= sub2;

end

sub2: begin

read_en  <=  4'd0;

write_en <= 16'b0010000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd2;

next     <= fetch1;

end

lshift1: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd3;

next     <= lshift2;

end

lshift2: begin

read_en  <=  4'd0;

write_en <= 16'b0010000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd3;

next     <= fetch1;

end

rshift1: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd4;

next     <= rshift2;

end

rshift2: begin

read_en  <=  4'd0;

write_en <= 16'b0010000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd4;

next     <= fetch1;

end

incac: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000110;

alu_op   <=  3'd0;

next     <= fetch1;

end

incdar: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00001010;

alu_op   <=  3'd0;

next     <= fetch1;

end

incr1: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00010010;

alu_op   <=  3'd0;

next     <= fetch1;

end

incr2: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00100010;

alu_op   <=  3'd0;

next     <= fetch1;

end

incr3: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b01000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

loadim1: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= loadim2;

end

loadim2: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= loadim3;

end

loadim3: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000010000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

jump: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= jumpz2;

end

jumpz1: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

if (z == 1)

next <= jumpz2;

else

next <= jumpz6;

end

jumpnz1: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000010;

alu_op <=  3'd0;

if (z == 0)

next <= jumpz3;

else

next <= jumpz7;

end

jumpz2: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000000010;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= jumpz3;

end

jumpz3: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000000010;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= jumpz4;

end

jumpz4: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000001000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= jumpz5;

end

jumpz5: begin

read_en  <=  4'd10;

write_en <= 16'b0000000000001000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= fetch3;

end

jumpz6: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= jumpz7;

end

jumpz7: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en  <= 16'b00000010;

alu_op  <=  3'd0;

next    <= jumpz4;

end

nop: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000010;

alu_op   <=  3'd0;

next     <= fetch1;

end

endop: begin

read_en  <=  4'd9;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= endop;

end

default: begin

read_en  <=  4'd0;

write_en <= 16'b0000000000000000;

inc_en   <=  8'b00000000;

alu_op   <=  3'd0;

next     <= fetch1;

end

endcase

endmodule

### 6.3.5 Data memory

module data_memory (

input             clk, write_en,

input      [15:0] addr,

input      [7:0]  data_in,

output reg [7:0]  data_out

);

reg [7:0] ram [65535:0];

always @(posedge clk) begin

if (write_en == 1)

ram[addr] <= data_in;

else

data_out <= ram[addr];

end

endmodule

### 6.3.6 Instruction memory

module instr_memory(

input             clk, write_en,

input      [15:0] addr, instr_in,

output reg [15:0] instr_out

);

reg [15:0] ram [180:0];

initial begin

ram[0] = 8'd38;

ram[1] = 16'd257;

ram[2] = 8'd9;

ram[3] = 8'd16;

ram[4] = 8'd4;

ram[5] = 8'd8;

ram[6] = 8'd38;

ram[7] = 8'd2;

ram[8] = 8'd29;

ram[9] = 8'd12;

ram[10] = 8'd38;

ram[11] = 8'd1;

ram[12] = 8'd8;

ram[13] = 8'd16;

ram[14] = 8'd25;

ram[15] = 8'd4;

ram[16] = 8'd13;

ram[17] = 8'd16;

ram[18] = 8'd27;

ram[19] = 8'd4;

ram[20] = 8'd8;

ram[21] = 8'd20;

ram[22] = 8'd25;

ram[23] = 8'd13;

ram[24] = 8'd38;

ram[25] = 16'd256;

ram[26] = 8'd8;

ram[27] = 8'd16;

ram[28] = 8'd25;

ram[29] = 8'd4;

ram[30] = 8'd8;

ram[31] = 8'd20;

ram[32] = 8'd25;

ram[33] = 8'd13;

ram[34] = 8'd38;

ram[35] = 16'd256;

ram[36] = 8'd8;

ram[37] = 8'd16;

ram[38] = 8'd27;

ram[39] = 8'd4;

ram[40] = 8'd8;

ram[41] = 8'd20;

ram[42] = 8'd25;

ram[43] = 8'd8;

ram[44] = 8'd38;

ram[45] = 8'd1;

ram[46] = 8'd29;

ram[47] = 8'd8;

ram[48] = 8'd19;

ram[49] = 8'd25;

ram[50] = 8'd12;

ram[51] = 8'd38;

ram[52] = 16'd257;

ram[53] = 8'd8;

ram[54] = 8'd16;

ram[55] = 8'd25;

ram[56] = 8'd4;

ram[57] = 8'd13;

ram[58] = 8'd16;

ram[59] = 8'd27;

ram[60] = 8'd4;

ram[61] = 8'd8;

ram[62] = 8'd20;

ram[63] = 8'd25;

ram[64] = 8'd13;

ram[65] = 8'd38;

ram[66] = 8'd255;

ram[67] = 8'd8;

ram[68] = 8'd16;

ram[69] = 8'd25;

ram[70] = 8'd4;

ram[71] = 8'd8;

ram[72] = 8'd20;

ram[73] = 8'd25;

ram[74] = 8'd13;

ram[75] = 8'd38;

ram[76] = 8'd255;

ram[77] = 8'd8;

ram[78] = 8'd16;

ram[79] = 8'd27;

ram[80] = 8'd4;

ram[81] = 8'd8;

ram[82] = 8'd20;

ram[83] = 8'd25;

ram[84] = 8'd8;

ram[85] = 8'd19;

ram[86] = 8'd25;

ram[87] = 8'd8;

ram[88] = 8'd38;

ram[89] = 8'd4;

ram[90] = 8'd31;

ram[91] = 8'd12;

ram[92] = 8'd38;

ram[93] = 16'd257;

ram[94] = 8'd8;

ram[95] = 8'd16;

ram[96] = 8'd27;

ram[97] = 8'd14;

ram[98] = 8'd19;

ram[99] = 8'd22;

ram[100] = 8'd38;

ram[101] = 16'd65278;

ram[102] = 8'd8;

ram[103] = 8'd16;

ram[104] = 8'd27;

ram[105] = 8'd41;

ram[106] = 8'd127;

ram[107] = 8'd38;

ram[108] = 8'd253;

ram[109] = 8'd8;

ram[110] = 8'd17;

ram[111] = 8'd27;

ram[112] = 8'd41;

ram[113] = 8'd118;

ram[114] = 8'd35;

ram[115] = 8'd36;

ram[116] = 8'd49;

ram[117] = 8'd3;

ram[118] = 8'd35;

ram[119] = 8'd35;

ram[120] = 8'd35;

ram[121] = 8'd38;

ram[122] = 8'd0;

ram[123] = 8'd10;

ram[124] = 8'd49;

ram[125] = 8'd3;

ram[126] = 8'd51;

ram[127] = 8'd38;

ram[128] = 8'd0;

ram[129] = 8'd9;

ram[130] = 8'd10;

ram[131] = 8'd11;

ram[132] = 8'd16;

ram[133] = 8'd4;

ram[134] = 8'd12;

ram[135] = 8'd18;

ram[136] = 8'd14;

ram[137] = 8'd19;

ram[138] = 8'd22;

ram[139] = 8'd16;

ram[140] = 8'd8;

ram[141] = 8'd38;

ram[142] = 16'd65278;

ram[143] = 8'd27;

ram[144] = 8'd41;

ram[145] = 8'd172;

ram[146] = 8'd17;

ram[147] = 8'd8;

ram[148] = 8'd38;

ram[149] = 8'd254;

ram[150] = 8'd27;

ram[151] = 8'd41;

ram[152] = 8'd160;

ram[153] = 8'd36;

ram[154] = 8'd36;

ram[155] = 8'd35;

ram[156] = 8'd35;

ram[157] = 8'd37;

ram[158] = 8'd49;

ram[159] = 8'd132;

ram[160] = 8'd38;

ram[161] = 8'd0;

ram[162] = 8'd10;

ram[163] = 8'd38;

ram[164] = 16'd258;

ram[165] = 8'd8;

ram[166] = 8'd16;

ram[167] = 8'd25;

ram[168] = 8'd9;

ram[169] = 8'd37;

ram[170] = 8'd49;

ram[171] = 8'd132;

ram[172] = 8'd51;

end

always @(posedge clk) begin

if (write_en == 1)

ram[addr] <= instr_in;

else

instr_out <= ram[addr];

end

endmodule

### 6.3.7 Top module

module machine(

input          clk,

input  [1:0]   status,

input  [7:0]   data_in,

input  [15:0]  data_addr_in,

output         end_process,

output [7:0]   data_out

);

wire        dm_en;

wire        im_en;

wire [15:0] pc_out;

wire [15:0] dar_out;

wire [15:0] bus_out;

wire [7:0]  dm_out;

wire [15:0] im_out;

assign data_out = dm_out;

processor    processor    (.clk(clk),.dm_out(dm_out),.im_out(im_out),.data_in(data_in),.data_addr_in(data_addr_in),.status(status),

.dm_en(dm_en),.im_en(im_en),.pc_out(pc_out),.dar_out(dar_out),.bus_out(bus_out),.end_process(end_process));

data_memory  data_memory  (.clk(clk),.write_en(dm_en),.addr(dar_out),.data_in(bus_out),.data_out(dm_out));

instr_memory instr_memory (.clk(clk),.write_en(im_en),.addr(pc_out),.instr_in(bus_out),.instr_out(im_out));

endmodule

### 6.3.8 Processor

module processor (

input        clk,

input [7:0]  dm_out, data_in,

input [15:0] im_out, data_addr_in,

input [1:0]  status,

output reg        dm_en, im_en,

output     [15:0] pc_out,

output reg [15:0] dar_out, bus_out,

output            end_process

);

wire [2:0]  alu_op;

wire [15:0] alu_out;

wire [15:0] dar_wire;

wire [15:0] bus_wire;

wire [15:0] regr_out;

wire [15:0] regr1_out;

wire [15:0] regr2_out;

wire [15:0] regr3_out;

wire [15:0] regr4_out;

wire [15:0] regr5_out;

wire [15:0] ac_out;

wire [15:0] ir_out;

wire [15:0] write_en ;

wire [3:0]  read_en;

wire [7:0]  inc_en;

wire        z;

regr    reg_r    (.clk(clk), .write_en (write_en[5]),.data_in(bus_wire),.data_out(regr_out ));

regrinc regr_r1  (.clk(clk), .write_en (write_en[6]),.data_in(bus_wire),.data_out(regr1_out),

.inc_en(inc_en[4]));

regrinc regr_r2  (.clk(clk), .write_en (write_en[7]),.data_in(bus_wire),.data_out(regr2_out),

.inc_en(inc_en[5]));

regrinc regr_r3  (.clk(clk), .write_en (write_en[8]),.data_in(bus_wire),.data_out(regr3_out),

.inc_en(inc_en[6]));

regr    regr_r4  (.clk(clk), .write_en (write_en[9]),.data_in(bus_wire),.data_out(regr4_out ));

regr    regr_r5  (.clk(clk), .write_en (write_en[10]),.data_in(bus_wire),.data_out(regr5_out ));

regrinc dar      (.clk(clk), .write_en (write_en[2]),.data_in(bus_wire),.data_out(dar_wire),.inc_en(inc_en[3]));

regr    ir       (.clk(clk), .write_en (write_en[3]),.data_in(bus_wire),.data_out(ir_out));

bus     bus1     (.r1(regr1_out ),.r2(regr2_out ),.r3(regr3_out ),.r4(regr4_out ),.r5(regr5_out ),.r(regr_out ),

.dar(dar_out),.ac(ac_out),.dm(dm_out),.im(im_out),.bus_out(bus_wire),.read_en(read_en),.clk(clk));

ac      ac1      (.clk(clk), .write_en (write_en[4]),.data_in(bus_wire),.data_out(ac_out),.alu_out(alu_out),

.alu_to_ac (write_en[13]),.inc_en(inc_en[2]));

regrinc pc       (.clk(clk), .write_en (write_en[1]),.data_in(bus_wire),.data_out(pc_out),.inc_en(inc_en[1]));

alu     alu1     (.clk(clk),.in1(regr_out ),.in2(ac_out),.alu_op(alu_op),.alu_out(alu_out),.z(z));

control control1 (.clk(clk),.z(z),.instruction (ir_out),.alu_op(alu_op),.write_en (write_en ),

.read_en(read_en),.inc_en(inc_en),.end_process (end_process ),.status(status));

always @ (posedge clk)

if (status == 2'b01) begin

dm_en   <= write_en [11];

im_en   <= 1'b0;

dar_out <= dar_wire;

bus_out <= bus_wire;

end

else if (status == 2'b10) begin

dm_en   <= 1'b1;

im_en   <= 1'b0;

bus_out <= data_in;

dar_out <= data_addr_in;

end

else if (status == 2'b11) begin

dm_en   <= 1'b0;

im_en   <= 1'b0;

dar_out <= data_addr_in;

end

endmodule

### 6.3.9 Registers without increment

module regr (

input             clk, write_en,

input      [15:0] data_in,

output reg [15:0] data_out

);

always @(posedge clk) begin

if (write_en == 1)

data_out <= data_in;

end

endmodule

### 6.3.10 Registers with increment

module regrinc (

input             clk, write_en, inc_en,

input      [15:0] data_in,

output reg [15:0] data_out = 16'd0

);

always @(posedge clk) begin

if (write_en == 1)

data_out <= data_in;

if (inc_en == 1)

data_out <= data_out + 16'd1;

end

endmodule

### 6.3.11 Test bench

`timescale 1 ns / 1 ps

module tb_machine();

reg        clk;

reg [1:0]  status;

reg [15:0] addr;

reg [7:0]  data;

wire       end_process;

wire [7:0] out;

integer    f;

reg [7:0]  read_data [0:65535];

machine dut(clk, status, data, addr, end_process, out);

initial begin

clk = 0;

forever clk = #(50) ~clk;

end

initial begin

$readmemb("image.txt", read_data);

f = $fopen("output.txt", "w");

status = 2'b10;

end

integer i;

initial begin

for (i=0;i<256*256;i=i+1) begin

addr = i;

data = read_data[i];

#100;

end

status = 2'b01;

while (end_process != 1'b1) begin

#100;

end

status = 2'b11;

for (i=0;i<128*128;i=i+1) begin

addr = i;

#200

$fwrite(f, "%u\n", out);

end

$fclose(f);

end

endmodule

## 6.4 Generating images

### 6.4.1 By processor implementation

import cv2

import numpy as np

# Show the original image

inputImage = open("input.txt", "r")

content = inputImage.read()

contentList = content.split("\n")

inputImage.close()

intList = [int(x, 2) for x in contentList]

uint8List = [np.uint8(x) for x in intList]

image = np.array(uint8List)

imageIn = image.reshape(256, 256)

print("Input image:\n", imageIn)

cv2.imshow('original', imageIn)

cv2.waitKey()

# Show the filtered image

inputImage = open("output_filtered.txt", "r")

content = inputImage.read()

contentList = content.split("\n")

inputImage.close()

intList = [int(x, 2) for x in contentList]

uint8List = [np.uint8(x) for x in intList]

image = np.array(uint8List)

imageFiltered = image.reshape(256, 256)

print("\nFiltered image:\n", imageFiltered)

cv2.imshow('Filtered', imageFiltered)

cv2.waitKey()

# Show the downsampled image

inputImage = open("output_downscaled.txt", "r")

content = inputImage.read()

contentList = content.split("\n")

inputImage.close()

intList = [int(x, 2) for x in contentList]

uint8List = [np.uint8(x) for x in intList]

image = np.array(uint8List)

imageDownsampled = image.reshape(128, 128)

print("\nDownsampled image:\n", imageDownsampled)

cv2.imshow('Downsampled', imageDownsampled)

cv2.waitKey()

cv2.destroyAllWindows()

### 6.4.2 By Python implementation

import cv2

import numpy as np

# Show the original image

inputImage = open("input.txt", "r")

content = inputImage.read()

contentList = content.split("\n")

inputImage.close()

intList = [int(x, 2) for x in contentList]

uint8List = [np.uint8(x) for x in intList]

image = np.array(uint8List)

imageIn = image.reshape(256, 256)

print("Input image:\n", imageIn)

cv2.imshow('Original', imageIn)

cv2.waitKey()

# Do the filtering

currentPixel    = 257

currentRowPixel = 0

totalRowPixels  = 253

totalPixels     = 65278

while True:

middle          =   np.uint16(image[currentPixel])

right           =   np.uint16(image[currentPixel+1])

left            =   np.uint16(image[currentPixel-1])

bottom          =   np.uint16(image[currentPixel+256])

top             =   np.uint16(image[currentPixel-256])

bottomRight     =   np.uint16(image[currentPixel+257])

topLeft         =   np.uint16(image[currentPixel-257])

bottomLeft      =   np.uint16(image[currentPixel+255])

topRight        =   np.uint16(image[currentPixel-255])

storeLoc        =   currentPixel-257

image[storeLoc] =   np.uint8((middle*4 + (right+left+bottom+top)*2 + (bottomLeft+bottomRight+topLeft+topRight))/16)

if currentPixel ==  totalPixels:

break

elif currentRowPixel == totalRowPixels:

currentRowPixel  =  0

currentPixel     =  currentPixel+3

else:

currentRowPixel  =  currentRowPixel+1

currentPixel     =  currentPixel+1

# Show the filtered image

imageFiltered = image.reshape(256, 256)

print("\nFiltered image:\n", imageFiltered)

cv2.imshow('Filtered', imageFiltered)

cv2.waitKey()

# Do the downsampling

currentPixelLoc = 0

currentStoreLoc = 0

currentRowPixel = 0

totalRowPixels  = 254

totalPixels     = 65278

while True:

image[currentStoreLoc] = image[currentPixelLoc]

if currentPixelLoc == totalPixels:

break

elif currentRowPixel == totalRowPixels:

currentRowPixel = 0

currentPixelLoc = currentPixelLoc+258

currentStoreLoc = currentStoreLoc+1

else:

currentRowPixel = currentRowPixel+2

currentPixelLoc = currentPixelLoc+2

currentStoreLoc = currentStoreLoc+1

# Show the downsampled image

imageDownsampled = image[:16384].reshape(128, 128)

print("\nDownsampled image:\n", imageDownsampled)

cv2.imshow('Downsampled', imageDownsampled)

cv2.waitKey()

# Get the image downsampled by the processor

inputImage = open("output_downscaled.txt", "r")

content = inputImage.read()

contentList = content.split("\n")

inputImage.close()

intList = [int(x, 2) for x in contentList]

uint8List = [np.uint8(x) for x in intList]

image = np.array(uint8List)

imageDownsampledMachine = image.reshape(128, 128)

# Calculate and show the error

errorArray = imageDownsampled - imageDownsampledMachine

error = np.count_nonzero(errorArray, axis=None, keepdims=False)

print("\nError:", error)

cv2.waitKey()

cv2.destroyAllWindows()